Advisory

Circular

U.S Department of

Transportation

Federal Aviation

Administration

Subject:  Power Loss After Takeoff	Date:  02/19/26		AC No:  61-XX

Initiated by:  AFS-800	Change:  Original

This advisory circular (AC) provides guidance for pilots, flight instructors, and training organizations concerning decision-making and training practices associated with loss of engine power after takeoff.

## 1. PURPOSE AND SCOPE

### Purpose

This Advisory Circular (AC) provides guidance to pilots and flight instructors on decision making, pre-takeoff planning, and maneuvering considerations following a loss of engine power during takeoff or initial climb in single-engine airplanes. The intent of this AC is to reduce the risk of loss of control and improve survivability by emphasizing performance-based planning, energy management, and control of angle of attack (AOA) during both on-airport and off-airport emergency landing scenarios.

### Scope

This AC supplements existing FAA guidance related to takeoff performance, emergency procedures, and flight training. It does not replace manufacturer-approved procedures contained in the Airplane Flight Manual (AFM) or Pilot’s Operating Handbook (POH), nor does it mandate specific techniques or minimum altitudes for maneuvering following power loss. Instead, it provides a framework for evaluating the feasibility of emergency landing options—including landing straight ahead, maneuvering ahead of the wing line, or, in limited circumstances, turning back toward the departure runway—based on aircraft performance, environmental conditions, pilot proficiency, and pre-takeoff planning.

This AC addresses:

- The importance of pre-takeoff assessment and briefing for power-loss contingencies.

- How maintaining energy state and controlling angle of attack can prevent loss of control following power loss.

- Factors that influence both the risk and success of an emergency turn-back maneuver.

- Training guidance to develop the perceptual and motor skills required to safely transition from climb to glide and maneuver without loss of control.

- The use and limitations of Airplane Performance Systems (APS) and supporting tools as aids to pilot decision making.

This AC applies to pilots operating single-engine airplanes under part 91 and to instructors providing flight training in the same category and class. While many concepts discussed herein may be applicable to multiengine or turbine aircraft, those operations are outside the scope of this document.

## 2. TERMS, DEFINITIONS, AND CONVENTIONS

For the purposes of this Advisory Circular, the following terms, definitions, and conventions apply:

Aircraft Performance System (APS) - A system, tool, or application that uses aircraft performance data, configuration, environmental conditions, and operating assumptions to assess post-takeoff maneuvering feasibility following power loss. An APS may provide performance assessments prior to takeoff or update calculations in real time using measured aircraft state, position, and wind to depict achievable flight paths and decision points, including when and where a turn-back maneuver may be feasible.

Angle of Attack (AOA) - The angle between a fuselage reference line and the relative wind, as measured or inferred by the airplane’s sensing system. For the purposes of this AC, AOA is treated as an operational parameter directly related to stall onset and performance, independent of wing chord reference, airspeed, weight, or bank angle. Stall occurs at a specific, critical angle of attack regardless of airspeed, weight, or bank angle.

Energy Management - The deliberate control of how an airplane’s available energy—expressed as altitude and airspeed—is exchanged and expended through pitch, bank, and configuration. Angle of attack governs this exchange by determining lift, load factor, and stall margin, and therefore how efficiently remaining energy can be used while maintaining control, particularly during time-critical events such as power loss after takeoff.

Engine Failure After Takeoff (EFATO) - A subset of PLATO characterized by a complete or effectively complete loss of thrust during takeoff or initial climb.

On-Speed Angle of Attack - A small range of angle of attack that provides an optimal balance between lift capability and stall margin during maneuvering. In straight-wing, propeller-driven airplanes, on-speed AOA corresponds closely to maximum endurance (minimum sink) glide rather than maximum lift-to-drag ratio. On-speed AOA varies with flap configuration. Operationally, it is the AOA corresponding to VREF when flying a stable approach.

Loss After Takeoff (PLATO) - Any loss or degradation of engine power occurring during takeoff or initial climb, including both partial power loss and complete loss of thrust.

Reference Maneuvering Glide Speed (VRG) - For airplanes not equipped with an angle-of-attack indication, VRG is a practical airspeed substitute for on-speed AOA. VRG is defined as 1.3 times the stall speed for the current flap configuration. Flight test has shown this reference provides excess maneuvering capability while preserving approximately 1.5–2 degrees of angle-of-attack margin, provided stall warning is treated as a boundary condition. In many single-engine airplanes, VRG will fall within the range of approach or reference speeds published in the AFM/POH.   However, VRG, as defined here, is based on stall margin and maneuvering capability rather than landing performance criteria.

Stall Warning as a Boundary Condition - Throughout the maneuvers described in this AC, stall warning is treated as an operational limit rather than a target. Pilots are expected to respect and “honor” the stall warning by recognizing it as the boundary of usable maneuvering performance, and not a region for sustained flight. Intentional or sustained operation with stall warning activated has no maneuver performance margin. If stall warning is encountered, pilots should immediately reduce angle of attack by unloading the wing.

Total Energy State - The combined kinetic and potential energy of the airplane. The maneuvers described in this AC are designed to maintain a continuous total energy state without increasing time, workload, and stall risk.

Training Drag - The intentional use of aerodynamic drag—such as flap extension in fixed-pitch propeller airplanes or zero-thrust power settings in controllable-propeller airplanes—to approximate the glide angle and external sight picture associated with an actual engine-off condition. Training drag is a simulation aid, not a procedural requirement, and is used to improve realism while maintaining controllability and safety.

Zero-Thrust Condition (Controllable-Propeller Airplanes) - A power and propeller control setting that results in approximately zero net thrust, producing a glide performance and sight picture representative of an engine-off condition. When zero-thrust data are not provided in the AFM/POH, it must be established through flight testing at altitude prior to instructional use.

## 3. BACKGROUND

Loss of engine power during takeoff or initial climb is one of the most time-critical and unforgiving emergencies when operating a single-engine airplane. If a power loss occurs without sufficient runway remaining to stop, the pilot must transition immediately from takeoff and climb to an off-runway landing or another survivable landing option. In that moment, the pilot is managing a rapid shift in aircraft energy state, high workload, and limited time—often while close to the ground and in proximity to obstacles, rising terrain, or developed areas. The outcome is frequently determined not by checklist knowledge, but by whether the pilot maintains positive aircraft control, avoids an aerodynamic stall, and executes a simple, survivable plan.

Historically, guidance and training practices have tended to emphasize landing approximately straight ahead following power loss after takeoff, and discourage any return toward the departure runway (often referred to as “the impossible turn”). While continuing straight ahead is often the lowest-risk choice, it is not universally the least hazardous option in all operating environments. In some departure scenarios—particularly those involving populated areas and obstacles along the departure path—pilots may be tempted to maneuver back toward the airport environment. This temptation is compounded by incomplete understanding of aircraft turnback capability, overconfidence in ad hoc assessment after the failure, and training that does not adequately reflect the time compression, energy loss, and performance penalties present during an actual power-loss event.

The feasibility of maneuvering back toward the airport is not governed by a single altitude value and cannot be reliably assessed after the failure occurs. Turnback capability varies with multiple interacting factors including runway environment, aircraft performance and configuration, weight, density altitude, wind direction and velocity, climb profile prior to the failure, pilot reaction and startle time, bank angle and resulting load factor, glide performance, and the need to transition from a turning maneuver to a stabilized approach path. Some airplanes do not have sufficient climb or glide performance to support a turnback under most conditions; some may do so only within a narrow range of conditions; and some may be capable more consistently—provided the maneuver is planned and briefed prior to takeoff and the pilot has demonstrated proficiency in the specific make and model.

A major contributor to poor outcomes following power loss after takeoff is loss of control during maneuvering. If the pilot delays pitch reduction, maintains climb attitude, or continues to carry rudder or bank inputs from the takeoff, the transition from climb to glide may result in the aircraft being close to the stall angle of attack. The turnback scenario further elevates stall risk because increased bank angle increases load factor, which in turn increases stall speed and reduces stall margin. In these circumstances, maintaining adequate margin above critical angle of attack is a primary safety requirement. Reliance on indicated airspeed alone to manage stall margin during maneuvering can increase pilot workload because stall speed varies with aircraft weight and load factor. A properly calibrated angle of attack (AOA) indication can reduce workload and loss-of-control risk by providing direct feedback on stall margin across a range of attitudes, speeds, and load factors.  However, pilots must also be prepared to operate safely without an AOA indication by using available cues, including stall warning systems, and by maintaining disciplined energy management.

Training practices can unintentionally create negative habit patterns if simulated power-loss events do not accurately replicate the aerodynamic and workload conditions associated with actual power loss. Residual thrust, delayed power reduction, and unrealistic setup can produce artificially favorable glide paths and turn performance, leading pilots to overestimate capability. Effective training should therefore emphasize prompt transition to an appropriate glide attitude, disciplined outside-visual control, strict respect for stall warning indications as a limit, and maneuvering that is consistent with the airplane’s demonstrated performance.

This AC builds upon the risk management and aeronautical decision-making principles described in AC 61-89, which emphasize preflight planning, hazard recognition, and avoidance of time-critical decision making without adequate preparation. Consistent with that philosophy, this AC provides a performance and energy-based framework for pre-takeoff planning, briefing, and training related to power loss after takeoff, including considerations relevant to maneuvering options and the turnback maneuver. The intent is not to promote a turnback as a default response, but to improve survivability by emphasizing decisions made before takeoff, control of angle of attack and stall margin during maneuvering and providing training that accurately reflects real-world constraints.

## 4. CORE SAFETY PRINCIPLES

Loss of engine power during takeoff or initial climb creates an immediate and irreversible reduction in available energy. From that moment forward, pilot actions determine how the remaining energy is managed and whether the airplane remains controllable. The following principles apply regardless of airplane type, equipment, or operating environment and form the foundation for the guidance provided in this AC.

Maintain aircraft control as the first priority. Following a power loss, prompt reduction of pitch attitude will prevent AOA from increasing to the stall AOA. Delayed or insufficient pitch reduction, continuation of climb attitude, or retention of takeoff rudder or bank inputs can rapidly place the airplane near the stall boundary. Positive control of pitch, bank, and yaw must be established and maintained before any consideration of landing options or maneuvering.

Recognize that the event is governed by the Total Energy State, not geometry. After power loss, the airplane’s total energy is finite and continuously decreasing. Maneuvering, including turns, consumes energy in terms of speed and altitude. The feasibility of any landing option is determined by how effectively the pilot manages the remaining energy, not by whether a particular runway or landing area appears reachable. Decisions based on visual alignment or perceived distance alone are unreliable under time-compressed conditions.

Appreciate that survivability is often constrained by time rather than horizontal distance or turn geometry. Each phase of maneuvering consumes time during which the airplane must remain above the stall angle of attack. Maneuvers that require multiple transitions—such as an initial aggressive turn followed by pitch adjustments to increase or decrease as needed to maintain kinetic energy, will increase workload and compound the risk of reducing stall margin.

Recognize that real-time assessment is limited by human factors and can be unreliable in this event. The number of interacting variables, combined with time compression and startle factor, exceeds what can be consistently evaluated through visual estimation or mental calculation after power loss. Effective response therefore depends on a structured decision framework established prior to takeoff, whether implemented through documented planning, a formalized briefing, or an Airplane Performance System (APS).

Understand and maintain stall margin. An aerodynamic stall occurs at a specific angle of attack, independent of airspeed, altitude, or airplane configuration. Angle of attack must be maintained with a margin between what is needed for maneuvering and the stall angle of attack,   regardless of whether it is directly indicated to the pilot. During maneuvering flight, if bank angle is increased, load factor will increase, which in turn will reduce the available stall margin. Maintaining adequate stall margin while maneuvering is essential to preventing loss of control.

Acknowledge the effects of time compression and the startle factor. Power loss shortly after takeoff leaves little time for analysis or deliberation. Pilot reaction time, startle response, and task saturation consume altitude and energy before any maneuvering begins. Because meaningful analysis may not be possible once airborne, viable responses to power loss after takeoff must be considered and briefed prior to the start of the takeoff roll.

Accept that complexity increases risk. Landing generally ahead of the departure path typically requires fewer control inputs, less maneuvering, and lower workload than attempting to reverse course. Maneuvers that increase bank angle, require precise coordination, or demand rapid transitions between climb, glide, and turning flight elevate the risk of compromising stall margin. When energy, altitude, or mental preparation are insufficient, simplifying the maneuver improves survivability.

Base decisions on demonstrated performance and proficiency. The ability to maneuver following power loss is airplane-specific and pilot-specific. Assumptions based on generalized performance, rules of thumb, or anecdotal experience are unreliable. Only demonstrated aircraft performance and practiced pilot proficiency under realistic conditions provide a valid basis for evaluating maneuvering options following power loss.

These core safety principles apply equally to airplanes equipped with advanced avionics, and to those without. While instrumentation and decision-support tools may assist pilots in managing energy and stall margin, they do not alter the underlying physics or reduce the need for disciplined control and pre-takeoff planning. Subsequent sections of this AC build on these principles to provide guidance on planning, training, and execution following power loss after takeoff.

### 4.1 Energy Management in Power-Loss-After-Takeoff Events

Following loss of engine power after takeoff, the airplane enters a strictly energy-limited condition. No additional energy is available beyond what exists at the moment of power loss. From that point forward, the pilot’s actions determine how rapidly that remaining energy is expended and whether sufficient margin remains to maintain control and reach a survivable landing. In this context, energy consists of altitude and airspeed. Altitude represents stored potential energy, while airspeed represents kinetic energy. After power loss, total energy continuously decays as the airplane descends and maneuvers, even though altitude and airspeed may be traded between each other. Energy cannot be regained; it can only be converted or expended. Because time, altitude, and energy are inseparable in this event, inefficient energy use directly reduces survivability.

Pilots do not manage energy directly. Instead, following loss of engine power, they manage the rate over time at which the remaining energy is exchanged and expended through control of pitch, bank, and configuration. Angle of attack (AOA) governs this exchange. AOA determines lift, load factor, induced drag, and stall margin, and therefore how efficiently the remaining energy can be used to support maneuvering flight without loss of control. Excessive angle of attack, abrupt control inputs, or uncoordinated maneuvering accelerate the rate of energy loss while simultaneously reducing available stall margin.

During maneuvering flight, particularly when bank angle is increased, load factor rises and the wing must operate at a higher AOA to produce the required lift. This reduces stall margin and increases the rate at which altitude and airspeed (total energy) are consumed. Attempts to “optimize” geometry—such as tightening a turn late, making abrupt pitch corrections, or introducing additional maneuvering phases—consume time and energy while increasing workload and stall risk.

Effective energy management following PLATO therefore emphasizes maintaining a single, continuous total energy state. Maneuvers should be executed smoothly and deliberately, without segmented phases that require reacceleration, extension, or recovery. Maintaining on-speed angle of attack, or its airspeed equivalent when AOA indication is not available, allows the airplane to operate near its most efficient lift condition while preserving stall margin under g load.

This approach differs from traditional guidance to “pitch for best glide speed,” which optimizes glide range in steady, wings-level flight. Following PLATO, the problem is not one of maximizing distance, but of maintaining control and stall margin during time-critical maneuvering at low altitude. Maintaining on-speed angle of attack, or VRG when AOA indication is unavailable, prioritizes controllability, predictable load-factor response, and continuous energy expenditure without segmented transitions. Techniques that rely on aggressive initial turns, unloading to extend glide, or subsequent deceleration introduce multiple phases that increase time, workload, and exposure to loss of control, and are inconsistent with disciplined energy management in a power-loss-after-takeoff environment.

Because energy is finite and continuously decreasing, feasibility of any maneuver—including a turn-back toward the departure runway—is governed not by geometric alignment or perceived distance, but by whether sufficient energy remains to complete the maneuver while maintaining control. This determination cannot be made reliably in real time. It must be established before takeoff using performance-based planning that accounts for aircraft configuration, environmental conditions, pilot proficiency, and the expected energy costs of maneuvering.

Throughout this advisory circular, energy management is treated as a foundational discipline rather than a discrete technique. The planning guidance, execution principles, and training maneuvers that follow are designed to develop the pilot’s ability to recognize, preserve, and deliberately expend remaining energy through disciplined angle-of-attack control, coordination, and configuration management. Failure to manage energy effectively—most often through excessive angle of attack or delayed pitch reduction—remains the primary contributor to loss of control following power loss after takeoff.

## 5. PRE-TAKEOFF PLANNING

Effective response to Power Loss After Takeoff (PLATO) depends primarily on decisions made before the takeoff roll begins. Because time, altitude, and cognitive capacity are severely limited following power loss, pilots should not rely on real-time analysis to determine viable landing options. Instead, a structured pre-takeoff planning process is essential to establish clear decision boundaries and reduce ambiguity during a time-critical event.

### 5.1 Plan for Power Loss as a Time-Critical, Energy-Limited Event

Loss of power after takeoff is characterized by continuous energy loss and compression of the time needed to make accurate decisions. Altitude is lost not only during maneuvering, but also during pilot analysis and response, the transition from climb to glide, and turns necessary for alignment with a potential landing area. Because these altitude losses occur sequentially and cannot be recovered, the feasibility of any maneuver must be evaluated within a continuous sequence, rather than as single events.

Pre-takeoff planning should therefore consider the entire sequence of events following power loss, including pilot reaction and startle time, transition from climb attitude to a stabilized glide,
maneuvering with increased load factor, and final alignment for landing. Plans that rely on later turn correction or pitch changes to increase speed will introduce additional phases that increase workload and erode stall margin.

## Figure 1.  Factors affecting turn-back.

### 5.2 Identify and Evaluate Governing Factors

Post-takeoff options are governed by multiple interacting factors as shown in Figure 1. No single parameter—such as altitude, airspeed, or distance from the runway—is sufficient by itself to determine whether a successful maneuver can be achieved. Pilots should evaluate these factors prior to takeoff and recognize that unfavorable combinations can quickly eliminate options.

Key factors include, but are not limited to:

- Aircraft performance: weight and center of gravity, climb capability prior to power loss, glide performance, and the effect of gear and flap configuration.

- Environmental conditions: density altitude, wind direction and velocity, runway orientation, terrain and obstacles.

- Pilot factors: reaction and startle response, training and proficiency, demonstrated ability to manage angle of attack during maneuvering, and mental preparation prior to takeoff.

- Operational context: departure routing, availability and suitability of landing areas, traffic and airspace constraints.

These factors collectively constrain what is possible following power loss.  Pre-takeoff planning should assume conservative performance and recognize that actual performance may be dictated by the least favorable conditions. The feasibility of maneuvering back toward the departure runway is ultimately governed by the relationship between the airplane’s climb performance prior to power loss and its glide performance during the return, after accounting for maneuvering, transition, and reaction-time losses.

### 5.3 Use a Structured Decision Framework

Because a real-time assessment can be unreliable during a PLATO event, pilots should establish a structured decision framework prior to takeoff. This framework may be implemented using written planning, standardized briefing techniques, or an APS. Regardless of format, the framework should clearly identify which actions are acceptable and which are excluded following power loss.

WARNING

Informal mental estimates, generalized rules of thumb, or ad hoc judgment are not sufficient to accomplish a successful turnback maneuver.

A valid framework:

- Is established before the start of the takeoff roll.

- Is based on demonstrated aircraft performance and pilot proficiency.

- Accounts for realistic reaction time and energy loss.

- Can be flown without further analysis once airborne.

Options outside a pre-briefed framework should not be attempted following power loss.

### 5.4 Conduct a Simple, Explicit Pre-Takeoff Brief

Pre-takeoff planning is most effective when it results in a concise verbal or mental brief that can be recalled immediately after power loss. Briefs should be simple, directive, and limited to a small number of clearly defined actions. An example pre-takeoff briefing structure is: “If power is lost prior to liftoff, I will reject the takeoff. If power is lost after liftoff and adequate performance is not available to maneuver, I will lower the nose, maintain control, and land directly ahead. If the pre-briefed conditions are met, a return toward the departure runway will be feasible.”  The structure of the briefing should be adapted to the aircraft and operating environment to provide an unambiguous plan that does not require further decision making after power loss.

5.5 Reassess When Conditions Change

The pre-takeoff briefing will not be the same for every takeoff.  It will change depending on aircraft loading, density altitude, wind, runway selection, or departure routing, all of which can significantly change post-takeoff options. Pilots should reassess and update their pre-takeoff plan when conditions differ from those previously evaluated. A plan that is valid under one set of conditions may not remain valid under another. Use of a structured decision framework or APS can assist pilots in efficiently reassessing assumptions and executing an alternate plan when conditions change.

## 6. AIRCRAFT HANDLING FOLLOWING POWER LOSS AFTER TAKEOFF

Loss of engine power after takeoff requires immediate and disciplined pilot action to maintain aircraft control and preserve remaining energy. Regardless of the landing option selected during pre-takeoff planning, survivability depends on successfully managing the transition from powered climb to partial or unpowered flight. The following guidance addresses that transition and applies to all post-takeoff power-loss scenarios.

### 6.1 Immediate Recognition and Response

When engine power is lost during the initial climb, the pilot must immediately recognize the event and respond by lowering the nose to arrest the climb and prevent the airplane from approaching the stall AOA. Retaining climb pitch attitude, even briefly, can rapidly erode stall margin. At the same time, residual takeoff control inputs, particularly rudder, must be corrected to restore coordinated flight. Accident investigations and training experience indicate that loss of control frequently occurs during this initial transition period, when startle, delayed pitch reduction, and improper coordination coincide with minimal altitude and energy. Prompt, decisive control inputs during the first seconds following power loss are vitally important.

### 6.2 Transition from Climb to Stabilized Glide

Following pitch reduction, the pilot should establish a stable glide attitude appropriate for the airplane and configuration. This transition should be smooth and deliberate, avoiding abrupt control movements that could increase AOA and reduce stall margin. Establishing a stable glide early reduces cognitive workload and preserves energy for subsequent maneuvering or landing alignment. During this phase, the pilot should prioritize outside visual references to maintain attitude and coordination, using cockpit instruments as confirmation rather than primary control cues.

### 6.3 Managing Angle of Attack During Maneuvering

Throughout execution, maintaining adequate margin above the stall angle of attack is essential. Increased bank angle, abrupt control inputs, or uncoordinated flight will increase load factor and reduce stall margin. In airplanes not equipped with an AOA indicator, pilots must rely on disciplined pitch control, coordination, and stall warning cues to preserve stall margin. Reliance on indicated airspeed alone is insufficient during maneuvering because stall speed varies with load factor and configuration.

### 6.4 Avoiding Common Transition Errors

Common errors during execution following power loss after takeoff include:

- Delayed pitch reduction resulting in an excessive angle of attack.

- Retention of takeoff rudder required during climb leading to uncoordinated flight.

- Focusing on cockpit instruments instead of outside visual references.

- Abrupt or excessive control inputs during the transition to glide.

- Attempts to “salvage” the situation through unplanned maneuvering.

Avoiding these errors requires discipline, training, and adherence to the pre-takeoff plan established before brake release.

### 6.5 Commitment to the Pre-Briefed Plan

Pilot actions following power loss must remain consistent with the plan established prior to takeoff. Options that were not explicitly evaluated, briefed, and trained for should be avoided once airborne. If conditions encountered during execution deviate from those assumed during planning—such as unexpected energy loss, degraded controllability, or inability to meet anticipated maneuver performance—the pilot should abandon the planned return and transition immediately to the best available landing area consistent with maintaining control of the airplane.

### 6.6 Discipline Over Optimization

Execution should prioritize controllability and sufficient stall margin rather than geometric perfection or runway alignment. The objective is not to “make the runway,” but to maintain controlled flight within the airplane’s performance envelope and land under control at the lowest practical energy state. Pilots should recognize that hesitation, abrupt control inputs, or attempts to salvage marginal execution increase risk and may negate the benefits of prior planning and training.

## 6.7 Restart Attempt

## Any restart attempt should be performed from memory and remains a secondary priority to maintaining aircraft control and executing the pre-briefed plan. Restart attempts should not delay or interfere with pitch control, coordination, or maneuver execution. Consideration may be given to making an emergency radio call, but not at the expense of flying the airplane.

## 7. TRAINING, DEMONSTRATION, AND PROFICIENCY

Turn-back feasibility following loss of engine power after takeoff cannot be established through abstract guidance or generalized assumptions. It must be demonstrated through structured training maneuvers that deliberately replicate the energy state, workload, and constraints of an actual power-loss event, while maintaining appropriate safety margins. This section provides guidance to instructors on how to design, conduct, and assess such training in a disciplined and repeatable manner.

## 7.1 Instructor Role and Training Philosophy

Instruction related to PLATO and potential turn-back maneuvers requires a disciplined, evidence-based training philosophy. The instructor’s role is not to teach pilots how to “attempt” a return to the runway, but to determine—through structured training and demonstration—whether such a maneuver is feasible for a given aircraft, configuration, and pilot under defined conditions.

Instructors must recognize that post-takeoff power loss is a time-critical, airspeed and altitude limited event. Training must focus on pre-event planning, and post-event maneuvering.. Maneuvers described in this section are intended to establish aircraft-specific performance limits, pilot workload tolerance, and preserving stall margin preservation while maneuvering. They are conducted at a safe altitude for training and demonstration purposes and are not intended to be replicated at low altitude.

A fundamental responsibility of the instructor is to act as a risk manager. If training demonstrates that required margins cannot be preserved—due to aircraft performance, configuration, environmental conditions, or pilot proficiency—the instructor should explicitly identify the turn-back maneuver as a no-go option for that combination. Successful training for PLATO includes identifying when a maneuver should not be attempted.

Training should emphasize:

- Disciplined management of airspeed and altitude.

- Continuous control of angle of attack during maneuvering.

- Avoidance of segmented or corrective flight techniques.

- Strict adherence to pre-briefed execution criteria.

Instructors should ensure that all training is conducted using stabilized configurations, conservative assumptions, and clearly defined abort criteria. Optimization or “best-case” performance demonstrations are inappropriate.  Training outcomes should be based on repeatable, worst-case criteria (weight, wind, configuration, etc). Finally, instructors should reinforce that turn-back feasibility will change from flight to flight. Differences in aircraft loading, configuration, environmental conditions, or pilot proficiency must be briefed to provide the best chance of success.  Training outcomes must therefore inform, not replace, structured pre-takeoff planning and the use of decision aids or APS to support conservative decision making.

### 7.2 Safety Considerations for Training and Demonstration

Training intended to evaluate turn-back feasibility following power loss after takeoff involves elevated risk and must be conducted with deliberate safeguards. The following safety considerations apply to all instructional activities described in this section and are intended to reduce the likelihood of loss of control or runway excursion during training.

Training and demonstration of turn-back maneuvering should be conducted at altitude until proficiency has been established. Low-altitude practice should only be introduced after the instructor has confirmed consistent performance, preserved stall margin, and disciplined execution. When conducted at low altitude, surface winds should be limited to a light and predictable value to reduce workload and alignment risk.

Instructors should recognize that a turn-back maneuver will likely result in a tailwind component for landing during training. Consideration should be given to conducting a low approach or go-around in lieu of a full-stop landing when tailwind conditions exist. Landing distance increases with a tailwind, and even modest tailwind components can materially affect rollout and directional control. Quartering tailwinds further increase the likelihood of loss of directional control, particularly with tailwheel-equipped aircraft.

When a return toward the runway is demonstrated, the initial turn should be made into the direction of any crosswind. Turning away from the wind during the initial turn greatly increases the turn radius and the distance required to realign with the runway. It also increases the likelihood of excessive bank, uncoordinated flight, and loss of control during the subsequent alignment maneuver.

Use of aerodynamic drag during training may be employed to approximate engine-off glide performance and sight picture. Flap extension in fixed-pitch propeller airplanes or zero-thrust power settings in controllable-propeller airplanes can increase descent rate and steepen glide angle, improving realism. However, instructors must recognize that flap use during training may increase stall margin relative to an actual engine-off condition and should not be relied upon to represent worst-case performance. Instructors must anticipate the need for timely power application and configuration changes during go-around. To avoid imparting negative habit patterns, configuration changes to replicate engine-off glide performance should be managed by the instructor.

WARNING
The emergency turn-back maneuver is inherently hazardous, even when practiced in training with an operating engine. It requires precise control with reduced stall margin and is often conducted close to the ground with minimal margin for error. Loss of control following an aerodynamic stall remains the primary threat during attempted turn-back maneuvers.

## 7.3 Baseline Configuration, Zero-Thrust Establishment, and Training Drag

All training maneuvers described in this section require a clearly defined baseline condition that accurately represents the energy state of an airplane following loss of engine power after takeoff. Establishing this baseline is essential to ensure that observed performance, altitude loss, controllability, and pilot perception are representative and repeatable. Instructors must ensure that configuration, thrust state, and crew duties are deliberately defined prior to flight and stabilized as briefed prior to maneuver execution.

In most training airplanes equipped with fixed-pitch propellers, idle power provides a reasonable approximation of an engine-off condition. However, instructors should recognize the limitations of this approximation and apply training drag where appropriate to replicate engine-off sight picture and energy decay more accurately. In airplanes equipped with controllable-pitch propellers, additional steps may be required to establish a representative engine-off condition, as idle power may not correspond to zero thrust.

The purpose of establishing a zero-thrust baseline or applying training drag is not limited to improving numerical performance accuracy. A primary objective is to provide the pilot with a visual sight picture, pitch attitude, closure rate, and control response representative of an actual engine-off condition. Training conducted at idle power alone often produces a flatter glide path and reduced pitch demand that differ materially from a true power-loss event and may condition inappropriate perceptual cues. Baseline and training-drag configurations should therefore be selected to replicate the pitch attitude, energy decay, and control feel the pilot will encounter following an actual loss of power.

Because the maneuver set includes both at-altitude training and demonstration maneuvers and pattern-based maneuvers conducted at low altitude, instructors should establish baseline thrust and training-drag conditions at altitude whenever feasible, then apply those validated settings consistently during subsequent training, including traffic-pattern work, while maintaining safety of flight as the overriding priority.

### 7.3.1 General Configuration and Crew Duties

Training maneuvers should be conducted from stabilized flight conditions. Configuration changes used for training or demonstration purposes must be set intentionally before maneuver initiation and not introduced dynamically during execution unless the maneuver specifically requires configuration management as an instructional element.

To prevent development of unsafe habit patterns:

- Configuration changes used for training or demonstration purposes are instructor-managed tools, not emergency techniques to be executed by the student.

- Students should not be taught to initiate drag deployment or power changes as a pilot response following simulated power loss.

- Crew duties should be clearly defined, with the instructor responsible for managing configuration changes (when used), monitoring safety margins, and terminating the maneuver as required, while the student focuses exclusively on aircraft control, coordination, and energy management.

This separation of duties supports safety of flight and reduces the risk of conditioning students to make inappropriate configuration changes during actual emergencies.

## 7.3.2 Fixed-Pitch Propeller Airplanes

In airplanes equipped with fixed-pitch propellers, reducing the throttle to idle typically provides a reasonable approximation of an engine-off condition. However, instructors should recognize that residual thrust and propeller drag still vary with airspeed and configuration, affecting both glide performance and visual cues.

If engine-off glide performance data is not explicitly provided in the AFM/POH, instructors should validate baseline glide characteristics at altitude by:

- Establishing a stabilized glide at the intended airspeed or AOA-equivalent

- Confirming consistent sink rate, pitch attitude, and closure cues

- Verifying repeatability across multiple runs

## 7.3.3 Use of Training Drag in Fixed-Pitch Airplanes

Training drag may be introduced to replicate engine-off glide path geometry and sight picture while maintaining altitude margins during training and demonstration. This is commonly accomplished through flap extension or other configuration changes selected and managed by the instructor. For most airplanes equipped with flaps, more than one-half flap extension will be required to match engine-off glide angle with an idling engine.

When using training drag:

- The configuration should be set and stabilized prior to maneuver execution

- The same configuration should be used consistently for pilot calibration and comparison

- Students should not be taught to deploy drag devices as part of a post-failure reaction technique

Instructors operating fixed-pitch airplanes not equipped to provide training drag may still conduct the maneuver set using idle power, provided the differences in glide angle, pitch attitude, and closure rate relative to a true engine-off condition are explicitly briefed and understood.

## 7.3.4 Controllable-Pitch Propeller Airplanes

In airplanes equipped with controllable-pitch propellers, idle power does not reliably represent an engine-off condition. Depending on propeller blade angle and governor behavior, the propeller may produce residual thrust or aerodynamic drag, significantly distorting glide performance and visual cues. While the procedures in the following paragraphs are primarily applicable to controllable-pitch airplanes, the underlying objective—replicating true engine-off sight picture and energy decay—applies to all training airplanes. Before conducting training maneuvers intended to establish aircraft-specific performance characteristics or perceptual cues, instructors should determine a zero-thrust baseline for the AFM/POH recommended airspeed and configuration.

## 7.3.4.1 Establishing Zero Thrust at Altitude

If zero-thrust procedures are not specified in the AFM/POH, instructors may establish a zero-thrust condition through flight testing at altitude by:

- Stabilizing the airplane in a clean glide at the intended reference airspeed

- Maintaining constant pitch attitude and airspeed

- Slowly adjusting propeller RPM to identify the point at which sink rate is minimized

The zero-thrust condition is identified where small RPM changes in either direction increase sink rate, indicating that net propulsive force has been minimized. This RPM should be recorded and used consistently for subsequent training maneuvers and planning purposes.

## 7.3.5 Training Drag in Controllable-Pitch Propeller Airplanes

Once a zero-thrust baseline is established, instructors may introduce training drag—such as partial flap extension or landing gear deployment—to replicate engine-off glide geometry and sight picture during training and demonstration while maintaining appropriate safety margins.

As with fixed-pitch airplanes:

- Training drag should be set deliberately and stabilized prior to maneuver execution

- Configuration changes should not occur dynamically during the maneuver unless explicitly required as an instructional element

- Students should not be conditioned to manipulate configuration following simulated power loss as a reflex

## 7.3.6 Application to Pattern-Based Maneuvers and Safety Emphasis

Validated baseline and training-drag configurations established at altitude may be applied during traffic-pattern training where appropriate. For maneuvers conducted in the traffic pattern, instructors must ensure that:

- The maneuver is conducted within established training and operational limitations

- The student’s attention remains focused on aircraft control, angle of attack, and energy management

- The instructor retains authority to terminate the maneuver immediately if stability, spacing, or safety margins degrade

Accurate baseline and training-drag configuration are prerequisites for meaningful training outcomes. Safety of flight always takes precedence over training objectives.

## 7.4 Training Maneuvers

## 7.4.1 Warm-up:  Airspeed Decay During ELATO Demonstration

### Purpose

To demonstrate to the pilot how airspeed decays in a climb attitude during ELATO.

## Objective

## Observe the pitch attitude and trim required to maintain a VY climb and airspeed decay when power is lost in that attitude.

### Setup

## This maneuver should be conducted at a safe altitude that allows full stall recovery to be completed at or above 1,500 feet AGL. The airplane should be configured for takeoff and established in a stabilized climb at VY and full power, trimmed normally.

### Execution

- Establish trimmed climb at VY.  Note pitch attitude and visual references.

- From a stabilized climb at VY, simulate power loss by reducing power smoothly to idle.

- Maintain pitch attitude/visual references.

- Observe rate of airspeed decay until stall warning.

- Recover to on speed AOA or VRG.

## 7.4.2 Angle of Attack Recovery

### Purpose

To teach the pilot to promptly, smoothly, and correctly control angle of attack during a simulated loss of engine power after takeoff, and to transition from a climb attitude to a stabilized glide without inducing a stall or excessive energy loss.

### Objective

Demonstrate proficiency transitioning from takeoff power and climb attitude to a stable on-speed angle of attack, or Reference Maneuvering Glide Speed (VRG) when AOA indication is not available, while maintaining continuous control of total energy.

### Setup

This maneuver should be conducted at a safe altitude that allows full stall recovery to be completed at or above 1,500 feet AGL. The airplane should be configured for takeoff and established in a stabilized climb at VY and full power, trimmed normally. For airplanes equipped with a controllable-pitch propeller, RPM may be adjusted as appropriate for training realism and controllability. For airplanes with fixed-pitch propellers, the instructor should plan for later introduction of training drag as described below.

### Execution

- From a stabilized climb at VY, simulate power loss by reducing power smoothly to idle.

- The pilot should immediately adjust pitch to reduce angle of attack and capture on-speed AOA, or VRG (1.3 × stall speed for the current flap configuration) when AOA indication is not available.

- From a VY climb, this transition typically requires a substantial pitch reduction, often to approximately a nose-level attitude.

- If the airplane was climbing at a steeper-than-normal climb attitude, a more aggressive and immediate pitch reduction may be required to prevent rapid energy decay and stall onset.

- In some airplanes, the required pitch change may approximate a reversal of the original climb angle. For example, a 15-degree climb attitude may require an immediate transition to approximately −15 degrees of pitch to preserve a flying angle of attack.

- The pilot should continue to make small pitch adjustments as required to capture and maintain the target AOA or VRG, using outside visual cues and stall-warning awareness to preserve margin.

### Training Drag Progression

After initial proficiency is demonstrated in a clean, straight-ahead recovery, the instructor may progressively introduce training drag by extending flaps as power is reduced to idle. This increases the pitch change required, steepens the glide angle, and produces a sight picture more representative of an actual engine-off condition. The purpose of training drag is to provide the pilot with a realistic visual and pitch reference, not to change the target energy state. The pilot should continue to fly on-speed AOA or VRG despite the altered sight picture.

### Maneuver Progression

Once consistent proficiency is demonstrated in straight-ahead recovery, the maneuver may be repeated while transitioning into a banked turn up to 45 degrees, emphasizing:

- Continuous AOA control

- Coordinated flight

- Avoidance of stall warning during maneuvering under load

### Instructional Emphasis

When thrust is reduced, a properly trimmed airplane will naturally pitch toward its trimmed angle of attack. Pilots should be taught to relax control pressure initially, allowing the airplane to begin the transition, then actively adjust pitch and trim to capture the target AOA or VRG. This maneuver is fundamental to all subsequent turn-back training and should be practiced until the pilot can:

- Respond smoothly and immediately to power loss

- Establish the correct energy state without hesitation

- Maintain stall margin during maneuvering

## 7.4.3 Gliding 45-Degree Banked Turn

### Purpose

To teach the pilot to fly a stabilized, steeply banked gliding turn that minimizes altitude loss while maintaining continuous control of angle of attack and total energy.

### Objective

Demonstrate proficiency in a stabilized gliding turn at a bank angle up to 45 degrees, flown through a total of 360 degrees of heading change at on-speed angle of attack, or Reference Maneuvering Glide Speed (VRG) when AOA indication is not available.  Observe altitude loss during the turn.

### Setup

This maneuver should be conducted at an altitude sufficient to allow completion of a descending 360-degree gliding turn and full recovery at or above 1,500 feet AGL.

From cruise or climb, the airplane should be configured as follows:

- Power reduced smoothly to idle

- For controllable-pitch propeller airplanes, RPM set by the instructor to achieve the planned training condition

- On-speed AOA or VRG (1.3 × stall speed for the current flap configuration) established

- Training drag applied by the instructor as appropriate

- Airplane trimmed to relieve control pressures prior to maneuver entry

### Execution

- From a stabilized glide at the target AOA or VRG, smoothly establish a bank angle up to 45 degrees in the desired direction.

- As the turn is initiated, the airplane will initially tend to descend until sufficient elevator input is applied. The pilot should apply coordinated elevator input to stabilize the flight path angle while maintaining coordination.

- Maintain coordinated flight and adjust pitch as required to hold on-speed AOA or VRG, ensuring stall warning is not sustained.

- Continue the gliding turn through a full 360 degrees of heading change, maintaining a constant bank angle and continuous energy state.

- Upon completion of the turn, recover to wings level and note the total altitude lost during the maneuver.

- Repeat the maneuver in the opposite direction.

### Instructional Emphasis

A gliding 45-degree banked turn typically requires approximately 1.2 to 1.4 g to maintain on-speed AOA or VRG. Increased elevator input is therefore required to support the additional load factor and preserve stall margin during the maneuver.

The flight path will not stabilize immediately upon turn entry. Pilots should expect glide path variation during approximately the first 240 to 270 degrees of turn before becoming fully stabilized. Instruction should emphasize patience, anticipation, and continuous pitch adjustment rather than abrupt or corrective control inputs.

Visual cues are critical. Pilots should be taught to use cowling attitude cues and outside references to maintain consistent pitch attitude and bank angle throughout the maneuver. Reliance on cockpit instruments alone should be avoided.

### Performance Awareness

The altitude loss observed during this maneuver is aircraft-specific and configuration-dependent. Instructors should ensure pilots understand that this altitude loss represents a repeatable performance requirement rather than a best-case outcome and should be applied conservatively during pre-takeoff planning.

## 7.4.4 Gliding 45-Degree Banked Turn to Accelerated Stall

### Purpose

To teach the pilot to recognize, avoid, and recover from accelerated stalls encountered during gliding maneuvering, with particular emphasis on angle-of-attack control and coordination. A 45-degree banked turn is the optimum bank angle for minimum time to complete the turn. Minimum time in the turn minimizes altitude loss.

### Objective

Demonstrate proficiency recognizing and recovering from coordinated and uncoordinated accelerated stalls occurring during a steeply banked gliding turn, while maintaining aircraft control and minimizing altitude loss.

### Setup

This maneuver should be conducted at an altitude sufficient to allow full stall recovery to be completed at or above 1,500 feet AGL.

From cruise or climb, the airplane should be configured as follows:

- Power reduced smoothly to idle

- For controllable-pitch propeller airplanes, RPM set by the instructor to achieve the planned training condition

- Reference Maneuvering Glide Speed (VRG) established

- Training drag applied by the instructor as appropriate

- Airplane trimmed to relieve control pressures prior to maneuver entry

### Execution

- From a stabilized glide at the target AOA or VRG, establish a coordinated banked gliding turn up to 45 degrees.

- With the flight path vector remaining below the horizon, gradually increase elevator input to increase load factor while maintaining bank angle and coordination.

- Continue increasing angle of attack until an accelerated stall is encountered.

- At stall onset, immediately reduce angle of attack, maintain coordinated controls, and recover to a stabilized turning glide at on-speed AOA or VRG.

### Stall Recognition and Recovery

Initial demonstrations and practice should be conducted with coordinated rudder input. In a coordinated accelerated stall, onset is typically recognized when the nose stops tracking through the turn, indicating that the wing has exceeded critical angle of attack under load.

### Instructional Emphasis

Instructors should emphasize that accelerated stalls during gliding maneuvering are angle-of-attack events rather than airspeed events. Increased bank angle and load factor rapidly reduce stall margin, even when indicated airspeed appears adequate.

Recovery from all accelerated stalls requires prompt reduction of angle of attack, followed by reestablishment of coordinated flight and stabilization at on-speed AOA or VRG. Altitude loss during recovery should be observed and discussed.

This maneuver reinforces the critical importance of coordination, AOA discipline, and load management during any gliding maneuver or turn-back following power loss.

## 7.4.5 180-Degree Power-Off Approach and Landing

### Purpose

To teach the pilot to visually assess and control the flight path vector during gliding maneuvering flight and to arrive at a planned touchdown area with appropriate energy for landing following power loss.

### Objective

Demonstrate confidence and proficiency in conducting a 180-degree power-off approach and landing using training drag, while maintaining on-speed angle of attack or Reference Maneuvering Glide Speed (VRG) and preserving adequate stall margin.

### Setup

This maneuver is conducted in the airport traffic pattern and integrates the gliding and maneuvering skills developed in the preceding exercises.

From an appropriate pattern position:

- Power reduced smoothly to idle

- For controllable-pitch propeller airplanes, RPM set by the instructor to achieve the planned training condition

- On-speed AOA or VRG (1.3 × stall speed for the current flap configuration) established, with stall warning treated as a boundary condition

- Training drag applied by the instructor to replicate the glide angle and sight picture associated with an actual engine-off condition

- Airplane trimmed to relieve control pressures

### Execution

- Begin the maneuver from a position abeam the intended touchdown zone (TDZ) at an altitude and lateral distance sufficient to permit a continuous glide to landing without power.

- Maintain on-speed AOA or VRG throughout the maneuver, adjusting pitch to control flight path angle rather than indicated airspeed.

- Execute a coordinated 180-degree descending turn toward the runway, continuously evaluating glide path using outside visual references.

- Adjust bank angle, turn radius, and configuration as required to arrive over the TDZ with appropriate energy for a stabilized transition to landing.

After initial proficiency is demonstrated from a standard abeam position, the maneuver should be practiced from varying starting points relative to the TDZ to develop the pilot’s ability to judge glide capability, manage energy, and adapt to non-standard geometry.

### Instructional Emphasis

A three-degree glide path is significantly shallower than the power-off glide angle of most general aviation airplanes. An engine-off approach typically produces a glide angle two to three times steeper than a normal powered approach. As a result, power-off approaches require a distinctly different sight picture and use of external references than pilots may be accustomed to during normal landings.

Instruction should emphasize that success in this maneuver depends on:

- Accurate perception of the aircraft flight path

- Disciplined AOA or VRG control

- Avoidance of late corrections that require abrupt pitch or bank changes

The objective is not precision spot-landing performance, but consistent arrival at the touchdown area with stable energy and adequate stall margin, allowing a controlled transition to landing.

This maneuver reinforces the visual judgment, coordination, and energy-management skills required to execute a survivable landing following power loss after takeoff.

## 7.4.6 Low-Altitude Post Power-Loss Maneuvering

### Purpose

To provide structured practice in idle-power maneuvering at low altitude toward a suitable landing site following simulated power loss, using predetermined performance information and disciplined energy management.

### Objective

Demonstrate familiarity with the conduct of a power-off turn-back and alignment maneuver toward the departure runway or an alternate landing surface within the departure airport environment, only for airplanes and conditions where feasibility has been established in advance using a validated performance-based planning method.

Preconditions

This maneuver should only be conducted when all of the following conditions are met:

- Pre-takeoff planning and briefing indicate that a turn-back is feasible for the specific airplane, configuration, loading, and environmental conditions.

- Performance feasibility has been confirmed prior to takeoff using an

- APS, manufacturer performance data, POH/AFM charts, or other documented performance-based analysis capable of accounting for aircraft configuration, environmental conditions, and maneuvering losses.

- Aircraft performance characteristics and pilot proficiency have been previously established through the higher-altitude maneuvers described earlier in this section.

- The maneuver is conducted under controlled instructional conditions with clearly defined abort criteria.

If these conditions are not met, this maneuver should not be conducted.

Setup

Conduct a pre-takeoff briefing covering all aspects of the takeoff and the potential for engine failure during climb. Perform a normal takeoff and initial climb on runway heading. Prior to takeoff, a turn-back decision point must be established using a validated performance-based method that accounts for aircraft configuration, weight, density altitude, wind, and expected maneuvering losses. This may be accomplished using an APS, manufacturer performance data, POH/AFM charts, or other documented analysis tools capable of predicting turn-back feasibility under the prevailing conditions.  Be certain that sufficient runway is available for landing and rollout, considering existing wind conditions.

Ensure that the airport traffic pattern is clear of other traffic and that no aircraft are waiting behind for takeoff.  Announce the intention to practice a simulated engine failure with a turnback maneuver.

During the initial climb, the instructor should maintain awareness of the airplane’s progress relative to the pre-established decision point. When the preplanned feasibility criterion are met the instructor may initiate the maneuver.

### Execution

- Reduce power smoothly to idle.

- For controllable-pitch propeller airplanes, RPM should be set by the instructor to achieve the planned training condition.

- Immediately establish the optimum angle of attack or Reference Maneuvering Glide Speed (VRG), with stall warning treated as a boundary condition.

- Transition promptly into a gliding turn with a bank angle targeting 45 degrees but typically between 35 and 50 degrees, toward the direction of the prevailing crosswind.

- Training drag may be applied by the instructor as appropriate, and the airplane trimmed to relieve control pressures.

- Throughout the maneuver, control of angle of attack or VRG should take priority over precise bank-angle control.

- Continue the gliding turn until aligned with the departure runway or a suitable alternate landing surface within the airport environment.

- Once aligned, assess whether a power-off approach and landing is feasible based on observed glide performance, closure rate, and remaining energy.

- If feasible, maneuver to a low approach or touchdown with the appropriate energy. If not, discontinue the maneuver and recover with a normal go-around procedure.

### Instructional Emphasis

Once the airplane is aligned toward the airport environment, feasibility becomes visually apparent. Glide capability, closure rate, and available turning room can be assessed rapidly when the flight path is directed toward the intended landing area.

Instruction should emphasize that:

- The maneuver is based on pre-determined aircraft performance.

- The decision to turn back is made before takeoff, not after power loss.

- Bank angle is a tool rather than a target; energy and AOA control take priority.

- Maintaining stall margin is essential to avoiding an accelerated stall.

This maneuver is not intended to teach pilots to “attempt” a turn-back. Its purpose is to demonstrate—under controlled conditions—when a turn-back is possible and when it is not, reinforcing conservative, binary decision making.

### Safety Considerations

This maneuver represents the highest workload and lowest safety margin condition in the training sequence. It should be introduced only after proficiency is demonstrated in all preceding maneuvers and should be discontinued immediately if control margins, spacing, or energy state degrade.

## 8. Summary and Training Philosophy

Loss of engine power after takeoff is among the most time-critical and unforgiving events a pilot may encounter. The limited altitude, rapidly decaying energy state, and high workload situation leave little opportunity for analysis once airborne. For this reason, survivability depends primarily on decisions made before the takeoff roll begins, not on time-compressed problem-solving after the failure occurs.

The guidance and maneuvers presented in this advisory circular are intended to support structured pre-takeoff planning, conservative decision making, and disciplined energy management. They are not intended to suggest that a return to the departure runway is universally achievable. In many airplanes and operating conditions, continuing a general path directly ahead remains the safest and most appropriate response to a loss of power after takeoff.

Turn-back feasibility is not a matter of optimism, intuition, or geometry. It is governed by the interaction of aircraft performance, configuration, environmental conditions, pilot proficiency, and available time. Training should therefore focus on determining whether a maneuver is feasible, not on persuading pilots that it should be attempted. A successful training outcome may be a clear determination that a return is not viable for a given aircraft or pilot.

Throughout all training and operational contexts, instructors and pilots should emphasize:

- Strict adherence to pre-briefed decision boundaries.

- Management of altitude and airspeed energy rather than chasing airspeed alone.

- Continuous control of angle of attack to preserve stall margin.

- Avoidance of segmented or corrective maneuvers that increase workload and time.

Where available, the use of APS or other validated decision aids is strongly encouraged to support conservative, directive decisions. Such tools assist pilots in evaluating feasibility before takeoff and reduce reliance on real-time judgment during an emergency.

Ultimately, the objective of this guidance is not to teach a maneuver, but to significantly improve outcomes by avoiding loss of control.  Pilots who brief clearly, plan conservatively, and train deliberately are far better prepared to respond effectively to power loss after takeoff—whether that response involves maneuvering or accepting an off-airport landing. In all cases, maintaining control of the airplane and protecting occupants is the desired outcome.